Purpose

On day 1, we will use IMAC Ni2+ affinity purification for the purification of a human serum albumin (HSA) specific nanobody (Nb) called M79 from clarified supernatants of lysed E coli cells prepared by the instructors.

Exercise day 1

Before the exercise, instructor thaws the crude extract aliquots used on that day and clarify them again by centrifugation. Centrifuge the sample in 15 ml conical at the max speed of the centriguge-rotor combination.

Instructor mounts buffers A and B on AEKTA start and equilibrate the pumps and loop. Follow equilibration through conductivity at 50% B. System equilibration can be done the day before or in the morning. On day, the AEKTA is loaded with the 5000 ul loop.

At exercise

- Introduction to safety.

- Introduction to the AEKTA start. GRA will give an introduction to the instrument during the lecture. We elaborate on this introduction, especially the instrument control software and the injection valve.

- Mount the 1 ml Ni2+ pre-charged IMAC column and equilibrate the column in A-20 with the loop in load position. Set the flow to 1 ml/min. Use the A280 and conductivity to decide when equilibration is complete.

- Zero the UV detector.

- Each team gets a thawed 5 ml sample of clarified crude extract from instructor and first withdraws 50 ul in an eppendorf tubeendorf tube for later SDS-PAGE and Bradford analysis. Store this on ice.

- Stop the pump. Put the injection valve in “load” position. Load the loop with the 5 ml clarified crude extract you got from the instructor.

- Switch the injection valve to “inject”. Start the pump at 0.5 ml/min for 10 min where you inject the sample onto the column. When the A280 starts to increase, collect the run-through in a single 50 ml tube.

- Wash the IMAC column with A-20 until A280 is almost stabilized. Expect to use 15-20 column volumes. Use a flow of 1 ml/min. Collect the run-through in the same 50 ml tube you used in the prior step.

- During the prior wash step, the team discusses the logic behind method 1 and fill the fraction collector with 20 tubes.

- When the wash step is complete, switch the injection port to “load position”.

- Start method 1 using the control software.

- During the run of method 1, withdraw 5 ul from the 50 ul eppendorf tube of crude extract and dilute it into 45 ul A-20. Add 5 ul of this 10x diluted sample to 1000 ul Bradford reagent in an eppendorf tube. Vortex for 10 seconds and leave for 1 min. A blue colour should develop. If the blue colour is not obvious, use 25 ul from the 10x diluted sample instead. Mark the tube with 1000+x ul as “BS”. Adjust these numbers after the pilot.

- Prepare a Bradford control in which the same volume x ul of A-20 is added to 1000 ul Bradford reagent, vortex and incubate for 1 min. Mark the tube with 1000+x ul as “BC”.

- Set the baseline of the spectrophotometer with the BC sample. Measure next the A595 for the BS sample.

- Calculate how many ug of total protein were added to the Bradford sample using the standard curve provided here. Using the known dilutions, calculate the total protein concentration (mg/ml) in the crude extract.

- When method 1 fraction collection is complete, withdraw 5 ul (A280 > 1) or 10 ul (A280 < 1) from selected peak fractions for SDS-PAGE analysis. Fill to 15 ul with A-20. Add 10 ul 5X SDS-PAGE sample buffer.

- Based on the protein concentration from step 13, withdraw a volume containing 25 ug from the crude extract and place it in an eppendorf tube, add A-20 to 15 ul.

- Assuming that the protein concentration is the same in the RT tube collected from the IMAC column, repeat the procedure above for the in the run-through tube.

- To all tubes, add 10 ul 5x SDS-PAGE load buffer and boil the samples.

- Fill in the scheme in the XL sheet to document the different sample volumes used. Be sure to include one slot for the Mw standard and to load the samples in the order you note in the XL sheet.

- The instructor will show you how to load and run the SDS-PAGE gel and how to stain+destain. If necessary, due to time limitations, the gel can be left in destain solution ON.

- The fractions from the gradient with highest A280 are selected for dialysis. The maximum number of selected fractions is 5. The dialysis is started with help from instructor.

Day 2

Purpose. Cation-exchange purification of dialyzed IMAC pool using a salt gradient 50-500 mM NaCl at pH 6.0 on the 1 ml Capto-S column.

- The dialyzed pool is clarified by centrifugation in a 15 ml conical tube together with the instructor and then filtered manually through a 0.45 um syringe filter.

- Withdraw 50 ul from the filtered sample into an eppendorf tube for determination of protein concentration and SDS-PAGE analysis. Mark the eppendorf tube “load capto S”.

- The system is equilibrated at 5 ml/min. The inject port is switched to inject and washed into B-50 until stable conductivity. As there is no chemical absorbing A280, conductivity is used to follow the equilibration.

- The 1 ml capto-S column is mounted and equilibrated into B-50 until stable conductivity is observed at flow of 1 ml/min.

- Stop the pump. The filtered pool is loaded into the 500 loop while the injection port is in the “load position”.

- The injection port is switched to “inject position”. Start the pump and thereby inject the sample onto the capto-S column. When A280 starts to increase, collect the run-through into a single 50 ml tube. Continue to wash with B-50 until A280 stabilizes. Fill the fraction collector with 20 tubes.

- Stop the pump and switch the injection port to “load position”.

- Start method 2 to run the gradient and collect the fractions.

- While the gradient is running, determine A280 for “load capto S” using buffer B-50 to set the baseline of the nanodrop spectrophotometer. If A280 is > 1.5, take 5 ul and dilute into 20 ul B-50 and then measure A280. Calculate the protein concentration in the “load capto S” tube by assuming that if A280=1, the protein concentration is 1 mg/ml.

- For SDS-PAGE, take 10 ul from relevant fractions and fill to 15 with B-50. From the “load capto S” tube, take a volume corresponding to 5 ug and fill to 15 ul with B-50 for SDS-PAGE analysis. Also, take 15 ul from the run-through tube.

- Add 10 ul 5x SDS-PAGE load buffer to each tube and boil the samples.

- Run the SDS-PAGE gel, stain+destain

- While the SDS-PAGE is running, update all tables and reply to questions. (Report Q to be formulated by GRA).

- When the method 2 has ended, exchange the bottle A to C-150 and bottle B to MQ.

- Dismount the capto-S column and mount the 500 ul loop on the injection port. Do not mount the SEC column yet.

- Switch the injection port to “inject position” and wash with 0 %B at 5 ml/min until conductivity is stable.

- Stop the pump, mount the SEC column and start the ON equilibration using method 6, 100% A, flow=0.1 ml/min, 50 ml total.

- Measure A280 on the nanodrop for the fraction with maximum absorption in the Capto-S chromatogram using B-50 to set the baseline on the nanodrop spectrophotometer. Assume that the sample is pure M79 nanobody, calculate the M79 concentration using the molar extinction coefficient calculated by protparam directly from the M79 sequence, see the spreadsheet.

Day 3

Purpose. Analysis of M79 nanobody activity validating the ability to form complex with HSA using the SEC column. In addition, you will learn to estimate the molecular weight of a protein from a SEC chromatogram using a standard curve.

- Unmount any column mounted. Equilibrate the system in 100% A containing C-150 with the injection port in “inject position” with the 500 ul loop mounted at 5 ml/min. Bottle B contains MQ and is not used today.

- Stop the pump and mount the SEC column

- Start the flow at 0.5 ml/min 100% A, still with the injection port in “inject”. Run until the A280 has stabilized again or maximum of 15 min. Fill the fraction collector with 25 tubes.

- While this wash step takes place, prepare the sample for run 1 (HSA only). Add 50 ug of HSA to an eppendorf tube. Get the concentrated HSA from the instructor. Fill to 450 ul total with C-150.

- Stop the pump. Switch the injection port to “load” and load the 450 ul HSA solution into the 500 ul loop. Switch injection valve to “inject”

- Start method 3 SEC for the HSA sample. Notice that HSA may contain 5-10% dimer, so don’t be confused if a second small early peak appears.

- While the HSA sample is running, you prepare the next two samples. The sample for Run 2 (M79 Nb+HSA) should contain 50 ug HSA and 25 ug M79 from the capto-S peak fraction obtained day 2. Fill to 450 ul with buffer C-150. Run 3 sample should contain 25 ug M79 from the capto-S pool from day 2. Also in this case, you fill to 450 ul with buffer C-150. For both samples, check your calculations with the instructor before mixing.

- When the first run is completed, save the peak fractions from the first run and mark them. Empty the non-peak tubes for which there is no A280 signal by simply shaking them into the zinc and reinsert them into the fraction collector along with any new tubes needed.

- Run the two other samples (HSA+M79) and (M79) as you did for the HSA sample.

- As soon as the (HSA+M79) run is complete, from the peak samples of the HSA and HSA+M79 runs, withdraw 15 ul for SDS-PAGE analysis.

- Likewise, prepare a sample of input sample by withdrawing a volume containing 3 ug from the capto-S peak fraction. Add C-150 to reach a total of 15 ul.

- Add 10 ul SDS-PAGE load buffer to all the samples to be analyzed, boil and load SDS-PAGE gel.

- While the SDS-PAGE gel is running, secure your data from the three days of chromatography from AEKTA control PC such that each team member has a copy.

- Stain and destain gel. Secure snaps of all three gels. Instructor may upload gel pictures to BS.